# Operacion Nexo 5G/6G - Anexo de calculos

# Escenario A - Distrito financiero

1. Ruido termico: N = -93.990 dBm.

2. Sensibilidad: Sens = -76.990 dBm.

3. Perdida maxima: Lmax = 125.990 dB.

4. Radio por cobertura: Rcov = 0.553 km.

5. Trafico por usuario: Auser = 0.100 Erl.

6. Densidad de trafico: Akm2 = 250.000 Erl/km2.

7. Recomendacion final: N = 4 con Rdesign = 0.129 km.

# Escenario B - Festival global

1. Ruido termico: N = -93.990 dBm.

2. Sensibilidad: Sens = -86.990 dBm.

3. Perdida maxima: Lmax = 135.990 dB.

4. Radio por cobertura: Rcov = 3.412 km.

5. Radio por capacidad: Rcap = 0.225 km.

6. Etapa de splitting recomendada: S4 con R = 0.213 km.

# Supuestos aplicados

| assumption | applied_value | justification |

| --- | --- | --- |

| Reference target area | 1 km2 for both scenarios | The statement asks for the number of cells over a proposed target area but does not provide a fixed value. A common normalized 1 km2 area is assumed to compare cell density without bias. |

| Channel partition in scenario A | floor(100 / N) channels per site, then uniform split across 3 sectors | The statement requests N = 3, 4, 7 and three sectors per site, but it does not define a more detailed PRB scheduler. The uniform split is the simplest reproducible assumption. |

| Interference approximation | 2 first-tier interferers for sectorized cells and 6 for omnidirectional reference | This is the standard first-tier hexagonal approximation used to compare reuse strategies in teaching-oriented planning exercises. |

| Hexagonal geometry | Regular hexagonal cell area model | Needed to convert traffic-limited area into an equivalent cell radius and then into the number of required sites. |

| Modern radio context | OFDMA/LTE/5G design logic with adaptive MCS | The statement explicitly asks for a modern mobile-network framing even though the numerical exercise is simplified to link budget and Erlang B. |